Survey

Round Rock FSU 00952

Barrel

- Coffee brewer at front counter

- Ned check valve on heater and expansion tank

- Water main in wall at heater

- Double stack ice machine with hoses for feeds and everpure scale filter and backflow

- Te store has uptaste resin tank

- Prefilter on MF620CC? above the mop sink

- Square D

- Cut out part of the grid at veggie sink for swf

- Remove the old swf not in useable spot

- Tea brewer at back by electric panels with a testable backflow

- Soda has a testable backflow with hose feeding all carbs

- 2 ice machines so bigger softener

- MF620CC wee the existing filters are

- Coffee pro mf620 were the uptaste is

- Use the ¾” feed at uptaste for 1A pump

- 1A pump feed the 2 ice machines ¾ feed, then split to 40’ away

- New drink manifold

- Pump above the freezer on the left side

- Drain into the mop sink at the floor drain

- Remove the bottom shelf at rack next cooler door to the left for 2 small softeners

- 2 small softeners not room for big softener

- Air compressor at carbs for co2? No tank so its off/on

- CO2 tank at fresh coral 100’

- Prefilter & ice pro at machine