TECHNICAL SERVICE REPORT

Water Analysis

| Customer: | Location: | Date: | PO#: |
| --- | --- | --- | --- |
| Chick-fil-A | #2682 | 8/13/09 |  |
| Address: |  | Tested By: |  |
| 2801 Crow Valley Pkwy Laguna Niquel, CA 92677 |  | AR |  |
| Phone: | Email: | Selecto Contact: |  |

| Recommendation: |  |
| --- | --- |

##### ANALYTICAL RESULTS

| Evaluation of: | Golden Spoon | El Pollo Loco | Burger King | Wachovia | Recommended Max Levels |
| --- | --- | --- | --- | --- | --- |
| Fluoride, ppm | 1.0 | 1.1 | 1.1 | 0.9 | 2 ppm |
| Chloride, ppm | 70 | 71 | 72 | 69 | 250 ppm |
| Nitrite, ppm | ND | ND | ND | ND | 1 ppm as N |
| Nitrate, ppm | 0.4 | 0.4 | 0.4 | 0.4 | 10 ppm as N |
| Sulfate, ppm | 273 | 284 | 273 | 271 | 250 ppm |
| Sodium, ppm | 93 | 99 | 103 | 94 | 250 ppm |
| Chloramine | ND | Detected | Detected | ND | .5 ppm |
| Chlorine, ppm | Detected | Detected | Detected | Detected | Detectable |
| Alkalinity, ppm | 110 | 110 | 110 | 110 | 150 ppm |
| Total Dissolved Solids, ppm | 700 | 750 | 730 | 720 | 500 ppm |
| Hardness, ppm | 157 | 181 | 175 | 170 | 100 ppm |
| pH | 7.2 | 7.4 | 7.4 | 7.5 | 6.5 – 8.5 ppm |
| Turbidity | 0.00 | 0.00 | 0.00 | 0.00 | .05 NTU |

The information supplied is believed to be accurate and is only indicative of the water tested.  The testing laboratory is not state certified.

3980 Lakefield Court    Suwanee, GA 30024    678.475.0799    800.635.4017

www.selectoinc.com

sales@selectoinc.com