TECHNICAL SERVICE REPORT

Water Analysis

| Customer: | Location: | Date: | PO#: |
| --- | --- | --- | --- |
| Chick-fil-A | #2513 | 12/12/12 | 420780 |
| Address: |  | Tested By: |  |
| 2437 West Main St. Norman, IK. 73069 |  | AR |  |
| Phone: | Email: | Selecto Contact: |  |
| 405-574-1500 | 02153@chick-fil-a.com |  |  |

| Recommendation: |  |
| --- | --- |

##### ANALYTICAL RESULTS

| Evaluation of: | Tap Water Sample | Filtered Water Sample | Recommended Max Levels |
| --- | --- | --- | --- |
| Fluoride, ppm | 1.2 | 1.0 | 2 ppm |
| Chloride, ppm | 26 | 25 | 250 ppm |
| Nitrite, ppm | ND | ND | 1 ppm as N |
| Nitrate, ppm | ND | ND | 10 ppm as N |
| Sulfate, ppm | 41 | 41 | 250 ppm |
| Sodium, ppm | 44 | 41 | 250 ppm |
| Chloramine | 2.0 | ND | .5 ppm |
| Chlorine, ppm | 0.55 | ND | Detectable |
| Alkalinity, ppm | 90 | 85 | 150 ppm |
| Total Dissolved Solids, ppm | 250 | 220 | 500 ppm |
| Hardness, ppm | 205 | 201 | 100 ppm |
| pH | 8.2 | 7.8 | 6.5 – 8.5 ppm |
| Turbidity | 0.33 | 0.05 | .05 NTU |
| Iron, ppm | 0.03 | 0.03 | 0.3 ppm |
| Chromium, ppm | 0.04 | 0.01 | 0.1 ppm |
| Copper, ppm | 0.84 | 0.04 | 1.3 ppm |

The information supplied is believed to be accurate and is only indicative of the water tested.  The testing laboratory is not state certified.

3980 Lakefield Court    Suwanee, GA 30024    678.475.0799    800.635.4017

www.selectoinc.com

sales@selectoinc.com